Python Codes of Automation

# Code 1

import getpass import telnetlib HOST = "192.168.200.254" user = input("Username: ") password = getpass.getpass() tn = telnetlib.Telnet(HOST) tn.read_until(b"Username: ") tn.write(user.encode('ascii') + b"\n") if password: tn.read_until(b"Password: ") tn.write(password.encode('ascii') + b"\n") tn.write(b"enable\n") tn.write(b"we\n") tn.write(b"conf t\n") tn.write(b"interface loopback0\n") tn.write(b"ip address 1.1.1.1 255.255.255.255\n") tn.write(b"end\n") tn.write(b"exit\n") print(tn.read_all().decode('ascii'))

# Code 2

from netmiko import ConnectHandler

# Define device details

R1 = {

"device_type": "cisco_ios",

"host": "192.168.200.254",

"username": "we",

"password": "we",

"secret": "we",

} try:

net_connect = ConnectHandler(**R1)     net_connect.enable()

print(net_connect.send_command("show ip interface brief"))

net_connect.disconnect() except Exception as e:     print(f"Error: {e}")

# Code 3

import telnetlib host = "192.168.200.253" user = "we" password = "we" tn = telnetlib.Telnet(host) tn.read_until(b"Username: ") tn.write(user.encode('ascii') + b"\n") if password:

tn.read_until(b"Password: ")     tn.write(password.encode('ascii') + b"\n") tn.write(b"enable\n") tn.write(b"we\n") tn.write(b"conf t\n") tn.write(b"vlan 2\n") tn.write(b"exit\n") tn.write(b"vlan 3\n") tn.write(b"exit\n") tn.write(b"vlan 4\n") tn.write(b"exit\n") tn.write(b"end\n") tn.write(b"exit\n")

print(tn.read_all().decode('ascii'))

# Code 4

from netmiko import ConnectHandler

# Define device details devices = [

{

"device_type": "cisco_ios",    # IP address of R1

"host": "192.168.200.254",    # Username for SSH

"username": "we",             # Password for SSH

"password": "we",             # Enable secret password

"secret": "we",

},

{

"device_type": "cisco_ios",   # IP address of IOU1

"host": "192.168.200.253",    # Username for SSH

"username": "we",

"password": "we",

"secret": "we"

}

]

# Loop through devices and save configurations for device in devices:     try:

# Establish SSH connection         net_connect = ConnectHandler(**device)         net_connect.enable()

# Set terminal length to 0 to disable pagination         net_connect.send_command("terminal length 0")

# Get the hostname of the device

hostname = net_connect.find_prompt().strip("#")

# Pull running configuration

config = net_connect.send_command("show running-config")

# Save configuration to a file named after the hostname

filename = f"config_{hostname}.txt"         with open(filename, "w") as file:

file.write(config)

print(f"Configuration saved for {hostname} in

{filename}")

# Disconnect the session         net_connect.disconnect()     except Exception as e:         print(f"Failed to retrieve configuration from

{device['host']}: {e}")

# Code 5

from netmiko import ConnectHandler from ipaddress import ip_network

# Define device details devices = [

{

"device_type": "cisco_ios",

"host": "192.168.200.254",  # R1

"username": "we",

"password": "we",

"secret": "we",  # Enable secret

},

{

"device_type": "cisco_ios",

"host": "192.168.200.252",  # R2

"username": "we",

"password": "we",

"secret": "we",

}

]

# OSPF Configuration

ospf_process_id = 1  # OSPF process ID for device in devices:     try:

# Connect to the device

net_connect = ConnectHandler(**device)         net_connect.enable()

# Get the list of interfaces and their subnet masks         output = net_connect.send_command("show ip interface brief")

interfaces = output.splitlines()

# List to hold OSPF configuration commands         ospf_commands = [f"router ospf {ospf_process_id}"]         for line in interfaces[1:]:  # Skip the header line             if "up" in line and "up" in line.split()[-1]:  #

Check for active interfaces                 parts = line.split()                 ip_address = parts[1]                 if ip_address != "unassigned":               # Retrieve the subnet mask using "show run interface <interface>"

interface_name = parts[0]                     int_output =

net_connect.send_command(f"show run interface

{interface_name}")

for config_line in int_output.splitlines():                         if "ip address" in config_line:

_, ip, subnet_mask = config_line.split()[-3:]

# Calculate the network and wildcard mask

network = ip_network(f"{ip}/{subnet_mask}", strict=False)                             ospf_commands.append(                                 f"network

{network.network_address} {network.hostmask} area 0"

)

# Send OSPF configuration commands to the device         net_connect.send_config_set(ospf_commands)         print(f"OSPF configured on {device['host']}:")         for cmd in ospf_commands:

print(cmd)

# Save configuration         net_connect.save_config()         net_connect.disconnect()     except Exception as e:         print(f"Failed to configure OSPF on

{device['host']}: {e}")

Code 6

import getpass import telnetlib

HOST = "192.168.126.201" user = input("Username: ") password = getpass.getpass() tn = telnetlib.Telnet(HOST) tn.read_until(b"Username: ") tn.write(user.encode('ascii') + b"\n") if password:

tn.read_until(b"Password: ")  tn.write(password.encode('ascii') + b"\n") tn.write(b"enable\n") tn.write(b"we\n") tn.write(b"conf t\n")

for vlan_id in range(2, 12):  # This will create VLANs from

2 to 11

tn.write(f"vlan {vlan_id} \n".encode('ascii'))  # Create

VLAN

tn.write(f"name WE-{vlan_id} \n".encode('ascii'))  # Set

VLAN name tn.write(b"end\n") tn.write(b"exit\n")

print(tn.read_all().decode('ascii'))